HELPS Notes:

1. GSD - Ground Sampling Distance

2. Ortho-Rectification

Orthorectification uses a Digital Elevation Model (DEM) to compensate for terrain. In areas with tall buildings, an additional Digital Surface Model (DSM) (which includes buildings and vegetation) can further improve accuracy.

Entire pipeline from the satellite to the final land detection:

Step 1: What does the satellite actually see?

- It captures an image like a camera but the satellite records much more than the normal image.

For every image it also stores

Satellite position (GPS)

Satellite orientation (roll, pitch, yaw)

Camera focal length

Pixel size of the sensor

Time of capture

Viewing angle

This is called the sensor model.

Step 2: Why is Raw image wrong

With Change in the position and angle of the satellite image, the object under the image get distorted - This is called relief displacement

This problem is solved by Orthorectification

Step 3: Orthorectification:

Orthorectification removes the distortions and answers where the point is actually located on earth

we pixel(500,500) along with this we need Latitude, Longitude, Elevation

For the above things we need, Camera Geometry, Satellite position, terrain height (Comes from DEM)

DEM: Digital Elevation model - Its a division of a location into a grid which holds a number stating the height above the sea level

DSM: Digital Surface model

NDVI: Normalized Difference Vegetation Index : a simple map tool that measures plant health using light

Dense forest      0.8

Grass             0.5

Bare soil         0.1

Road             -0.1

Water            -0.3

Satellite imagery

DEM (Digital Elevation Model) (you wrote GEM)

DSM (Digital Surface Model) (you wrote GSM)

GIS (Geographic Information System) (perhaps you meant GES)

Together, these can absolutely identify candidate landing areas, but they cannot guarantee that a location is safe to land. They are excellent for screening and ranking potential sites.

Limitations:

difficult or impossible to detect reliably:

Thin power lines

Temporary vehicles

People

Loose debris

Recently erected structures

Ground firmness (e.g., mud beneath dry-looking soil)

Weather conditions at landing time

How to identify the surface type:

Step 1: Every surface reflects light differently

Sunlight contains many wavelengths.

Some light is:

- absorbed

- reflected

- transmitted

Step 2: Satellites don't only see RGB

Our eyes see only:

- Red

- Green

- Blue

Modern satellites measure many more bands.

For example, Sentinel-2 has:

| Band | Wavelength | Used for |
| --- | --- | --- |
| Blue | 490 nm | Water, atmosphere |
| Green | 560 nm | Vegetation |
| Red | 665 nm | Vegetation |
| Red Edge | 705–783 nm | Plant health |
| Near Infrared (NIR) | 842 nm | Vegetation |
| SWIR | 1610 nm | Soil moisture, built-up areas |
| SWIR2 | 2190 nm | Minerals, rocks |

Step 3: Different materials have different signatures

easuring reflectance (%) at different wavelengths.

Each has a characteristic spectral signature. A satellite pixel can be compared against these reference signatures to estimate what material is present.

Step 5: Classification

Suppose one pixel has:

Blue = 30

Green = 42

Red = 25

NIR = 220

SWIR = 70

A classifier might determine:

Very high NIR

↓

Healthy vegetation

Another pixel:

Blue = 18

Green = 17

Red = 15

NIR = 8

SWIR = 5

might be classified as water.

This can be done using:

- Rule-based thresholds

- Traditional machine learning (Random Forest, SVM)

- Deep learning models (U-Net, SegFormer, DeepLabV3+, etc.)

The Pole of Inaccessibility is the point inside a region that is farthest from every boundary or obstacle.

PoI = Center of the Largest Empty Circle inside the polygon