Research on AI in Aerospace - Helicopeter

- Predictive Maintenance:

Main Rotor Data

- Main Rotor RPM (Nr)

- Rotor Blade Track

- Rotor Blade Lag

- Rotor Balance

- Rotor Vibration

- Rotor Load

Tail Rotor Data

- Tail Rotor RPM

- Tail Rotor Vibration

- Tail Rotor Balance

Transmission Data

- Main Gearbox Temperature

- Main Gearbox Pressure

- Main Gearbox Vibration

- Intermediate Gearbox Vibration

- Tail Gearbox Vibration

- Transmission Torque

- Gear Wear Indicators

- HUMS systems continuously monitor gearbox and rotor health because these are among the most critical helicopter components

- HUMS stands for Health and Usage Monitoring System

- Standard helicopter Health and Usage Monitoring System (HUMS) is traditionally rule-based, using specific math formulas called Condition Indicators (CIs).

Current Procedure of HUMS

- Signal Processing: Accelerometers capture raw vibration signals from the gearboxes, bearings, and rotors.

- Time Synchronisation: A tachometer measures the exact speed of the rotating shaft. The system uses this to chop up the raw vibration signal so it matches exactly one full rotation of a specific gear or bearing. 

- Condition Indicator (CI) Calculation: The system applies pre-defined mathematical rules to this cleaned data to calculate a single number (a CI). For example:

  - FM4 (Factor Method 4): A rule that detects isolated tooth damage on a gear by analyzing peaks in the vibration wave.

  - NA4: A rule designed to catch progressive, widespread gear damage over time.

  - Crest Factor: A rule dividing the peak vibration level by the average level to spot sudden spikes from cracked bearings.

- Threshold Alerting: If a CI number crosses a strict, predetermined limit (e.g., if Vibration Amplitude > X), the system flags a warning for the maintenance crew.

How AI can help and how is it helping right now!

  - Anomaly Detection (Unsupervised Learning)

- Autoencoders, Isolation Forest

- Learns baseline working of a healthy helicopter

  - Remaining Useful Life (RUL) Prediction – Supervised Learning

- Time series models – RNN , LSTM, GRU , Transformer

- feed the AI historical data from past parts that actually broke.  AI learns the exact trajectory a vibration trend takes as a crack expands.

- Predicts the amount of life left for a part before breaking

  - Advanced Filtering using CNN

- Uses CNN

- look at vibration data transformed into a visual graph (a spectrogram).

- CNN can automatically filter out ambient cockpit noise, wind buffeting, and rotor blade movements to isolate the microscopic sound of a failing bearing deep inside the main gearbox.

  - Multi-Sensor Flight Path and Fuel Optimization

- consume immense fuel during complex maneuvers like hovering or low-altitude tactical routing.

- The Data Used: Real-time atmospheric sensors, tail rotor torque parameters, engine fuel-flow metrics, and GPS data.

- AI models recalculate optimal routes in real-time, drastically reducing fuel burn and carbon emissions by minimizing high-power drag scenarios

  - Automated Search and Rescue (SAR) Object Detection

- Data Used: 4K electro-optical cameras, Forward-Looking Infrared (FLIR) thermal sensors, and Synthetic Aperture Radar (SAR)

- deploy computer vision systems (such as the G4SAR software)

- Deep neural networks scan the sensor feed in real-time to pick up human beings or debris in dense brush or open water, spotting moving targets even if only a single limb is visible to the sensor

Under Research Applications

  - Emergency Landing Recommendation – using Computer Vision and Onboard Perception Systems

- Shield AI Hivemind, which are tested on platforms like the Airbus UH-72B Lakota helicopter

- It uses deep learning models to actively detect and filter out wires, trees, rocks, humans, and cars that a panicked pilot might miss in a split second.

- High-to-Low Resolution Neural Networks

  - The AI can take lower-resolution, basic geographic surface maps and use predictive modeling to determine if a hidden field is actually smooth enough for a landing. It builds a real-time 1"suitability area map" for the pilot

- Database Validation AI

  - Aviation authorities also use AI to scan the earth and build better emergency maps. Researchers at Rowan and Rutgers universities created a Convolutional Neural Network (CNN) that scans global satellite imagery to automatically verify coordinates of every hidden helipad, hospital roof, or flat clearing. This system actually expanded the Federal Aviation Administration (FAA) database by finding thousands of unmapped, safe landing options

  - Voise activated pilot guide during Emergency

Basic Idea of Project – Early Fault Detection / Prediction using Vibration Data obtained from the Helicopter

Maintenance Data

Ground crews generate:

- Fault Codes

- Maintenance Logs

- Component Replacement History

- Inspection Reports

- Oil Analysis Reports

- Pilot Defect Reports

- Unscheduled Maintenance Events

- Remaining Useful Life Estimates

For AI, these labels are often more valuable than the sensor data itself because they tell you what actually failed.

Project to think of “Helicopter [any part (depends on what vibration is linked the most) ] Health Monitoring using Vibration-Based Anomaly Detection”